# Networking Assignment Answers

ROUTING AND SWITCHING

## 1. Explain Switch

A switch is a networking device used to connect multiple devices such as computers, printers, and servers in a Local Area Network (LAN). It works on the Data Link Layer (Layer 2) of the OSI model and transfers data using MAC addresses.

When data comes to the switch, it checks the destination MAC address and sends the data only to the correct device. This improves network speed and reduces unnecessary traffic.

Advantages of Switch:
• Fast communication
• Reduces network collisions
• Better network performance
• Secure data transfer

Types of Switch:
1. Unmanaged Switch
2. Managed Switch
3. Layer 3 Switch

## 2. Explain Switch Boot Sequence

The switch boot sequence is the process followed when a Cisco switch starts after power on.

Steps of Boot Sequence:
1. POST (Power-On Self-Test)
   The switch checks hardware components such as CPU, RAM, and interfaces.

2. Load Boot Loader
   The boot loader program is loaded from ROM memory.

3. Initialize Flash File System
   Flash memory is checked and IOS file is located.

4. Load Cisco IOS
   The Cisco IOS operating system is loaded into RAM.

5. Load Startup Configuration
   The switch checks NVRAM for saved configuration.

6. Switch Ready
   Interfaces become active and the switch starts working.

## 3. Explain Three Methods to Access Switch Command Line Interface (CLI)

The Command Line Interface (CLI) is used to configure and manage a switch. There are three methods to access CLI.

1. Console Access
Console access uses a console cable to connect a PC directly to the switch console port. It is mainly used for initial configuration and does not require network connectivity.

2. Telnet Access
Telnet allows remote access to the switch using an IP address. It works on port 23. Telnet is not secure because data is sent in plain text.

Example:
telnet 192.168.1.1

3. SSH Access
SSH (Secure Shell) provides secure remote access to the switch. It encrypts the data and works on port 22.

Example:
ssh admin@192.168.1.1

Difference Between Telnet and SSH:
• Telnet is not secure
• SSH is secure and encrypted

## 4. Explain and Configuring the Cisco Internet Operating System (IOS)

Cisco IOS stands for Cisco Internetwork Operating System. It is the operating system used in Cisco routers and switches. IOS is responsible for managing networking functions, security, routing, and switching.

Cisco IOS Modes:

1. User EXEC Mode
This mode provides limited access to commands.
Example:
Switch>

2. Privileged EXEC Mode
This mode provides full access to configuration and monitoring commands.
Command:
enable

Example:
Switch#

3. Global Configuration Mode
This mode is used to configure device settings.
Command:
configure terminal

Example:
Switch(config)#

Basic IOS Configuration Commands:

Set Hostname:
Switch(config)# hostname S1

Configure Enable Password:
S1(config)# enable secret cisco

Configure Console Password:
S1(config)# line console 0
S1(config-line)# password admin
S1(config-line)# login

Configure IP Address:
S1(config)# interface vlan 1
S1(config-if)# ip address 192.168.1.2 255.255.255.0
S1(config-if)# no shutdown

Save Configuration:
S1# copy running-config startup-config

## 5. Explain Switch Port

A switch port is a physical interface on a switch used to connect devices such as computers, printers, routers, or other switches.

Each switch port can be configured with different settings like speed, duplex mode, and VLAN membership.

Types of Switch Ports:

1. Access Port
An access port belongs to a single VLAN and is mainly used to connect end devices like PCs.

Example:
Switch(config-if)# switchport mode access

2. Trunk Port
A trunk port carries traffic of multiple VLANs and is mainly used between switches.

Example:
Switch(config-if)# switchport mode trunk

Common Switch Port Commands:

Enable Port:
Switch(config-if)# no shutdown

Disable Port:
Switch(config-if)# shutdown

Set Port Speed:
Switch(config-if)# speed 100

Configure Duplex Mode:
Switch(config-if)# duplex full

Port Status:
• Up – Port is active
• Down – No connection
• Administratively Down – Port manually disabled

## 1. OSPF Neighbor Relationship Question

The complete configuration in the exhibit is not clearly visible in the screenshot, so the exact answer cannot be determined accurately. Please share a clearer image of the configurations of R1, R2, R3, and R4 for the correct answer.

## 3. enable secret [password] is hashed using which algorithm?

Answer: A. MD5

## 4. - An engineer connects to Router R1 and issues a show ip ospf neighbor command. The status of neighbor 2.2.2.2 lists FULL/BDR. What does the BDR mean?

## Answer: D. Router 2.2.2.2 is a backup designated router.

## 5. Which command is used to view the neighbor discovery table on a PC?

Answer: C. netsh interface ipv6 show neighbor

## 6. What type of variable is shown? Routers = [R1,R2,R3]

Answer: A. List

## 7. Identify the fields in an IPv4 header. (Choose three)

Answers:
B. Time to Live
C. Source address
D. Destination address