Dhruti Pandya

Assignment: module -5 Network Fundamentals and Building

Networks

Section 1: Multiple Choice

1. What is the primary function of a router in a computer network?

a) Assigning IP addresses to devices

b) Providing wireless connectivity to devices

c) Forwarding data packets between networks

d) Managing user authentication and access control

Ans:c) Forwarding data packets between networks

2. What is the purpose of DHCP (Dynamic Host Configuration Protocol) in a

computer network?

a) Assigning static IP addresses to devices

b) Resolving domain names to IP addresses

c) Managing network traffic and congestion

d) Dynamically assigning IP addresses to devices

Ans:d) Dynamically assigning IP addresses to devices

3. Which network device operates at Layer 2 (Data Link Layer) of the OSI model

and forwards data packets based on MAC addresses?

a) Router

b) Switch

c) Hub

d) Repeater

Ans:b) Switch

4. Which network topology connects all devices in a linear fashion, with each

device connected to a central cable or backbone?

a) Star

b) Bus

c) Ring

d) Mesh

Ans: b) Bus

Section 2

True or False: A VLAN (Virtual Local Area Network) allows network administrators to logically segment a single physical network into multiple virtual networks, each with its own broadcast domain.

Answer: True

True or False: TCP (Transmission Control Protocol) is a connectionless protocol that provides reliable, ordered, and error-checked delivery of data packets over a network.

Answer: False

True or False: A firewall is a hardware or software-based security system that monitors and controls incoming and outgoing network traffic based on

predetermined security rules

Answer: True

8. Describe the steps involved in setting up a wireless network for a small office or home office (SOHO) environment.

Steps to Set Up a Wireless Network for a SOHO (Small Office / Home Office)

1.Plan the Network Requirements

Decide how many devices will connect (PCs, laptops, mobiles, printers).

Check internet speed requirement.

Choose suitable location for router (center of office/home).

2. Choose Required Hardware

Modem – Connects to Internet Service Provider (ISP).

Wireless Router – Provides Wi-Fi connection.

(Optional) Switch – For more wired connections.

(Optional) Wi-Fi Extender – For large area coverage.

3. Connect the Devices

Connect modem to ISP line.

Connect router to modem using Ethernet cable (WAN port).

Power on modem and router.

Connect computer to router (Wi-Fi or LAN cable).

4. Configure the Router

Open browser and enter router IP address (like 192.168.1.1).

Login using default username & password.

Change default admin password.

Set Wi-Fi Network Name (SSID).

Set strong Wi-Fi password.

5. Enable Security Settings

Use WPA3 or WPA2 encryption.

Disable WPS (if not needed).

Enable firewall.

Update router firmware.

6. Configure Network Settings

Enable DHCP (automatic IP assignment).

Set IP address range (example: 192.168.1.2 – 192.168.1.100).

Configure DNS (automatic or Google DNS 8.8.8.8).

7. Connect Wireless Devices

Search Wi-Fi name (SSID).

Enter password.

Test internet connection.

8. Test and Optimize

Check signal strength.

Place router at higher position.

Avoid walls and electronic interference.

Use Wi-Fi extender if needed.

Section 4: Practical

9. Demonstrate how to configure a router for Internet access using DHCP

(Dynamic Host Configuration Protocol).

Step 1: Configure WAN Interface to Obtain IP via DHCP

Router> enable

Router# configure terminal

Router(config)# interface gigabitEthernet0/0

Router(config-if)# ip address dhcp

Router(config-if)# no shutdown

Router(config-if)# exit

Step 2: Configure LAN Interface

Next, configure the LAN interface with a static IP address.

Router(config)# interface gigabitEthernet0/1

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Step 3: Configure DHCP Server on Router

Now, configure the router to assign IP addresses to devices in the LAN network.

Router(config)# ip dhcp pool OFFICE

Router(dhcp-config)# network 192.168.1.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.1.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

Step 4: Exclude the Gateway Address

To prevent the router’s IP address from being assigned to clients:

Router(config)# ip dhcp excluded-address 192.168.1.1

Step 5: Configure NAT for Internet Access

Network Address Translation (NAT) allows private IP addresses to access the Internet.

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

Router(config)# ip nat inside source list 1 interface gigabitEthernet0/0 overload

Router(config)# interface gigabitEthernet0/0

Router(config-if)# ip nat outside

Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1

Router(config-if)# ip nat inside

Router(config-if)# exit

Step 6: Save Configuration

Router# write memory

Section 5:

- Discuss the importance of network documentation in the context of building and managing networks.

Network documentation means keeping a written record of all network details such as devices, IP addresses, configurations, topology, and security settings.

It is very important for building and managing networks for the following reasons:

1. Easy Troubleshooting

If a problem occurs, documentation helps identify:

Device location

IP addresses

Cable connections

Network configuration

This saves time and reduces downtime.

2. Better Network Management

Administrators can easily:

Monitor devices

Update configurations

Manage IP addresses

Track changes

It keeps the network organized.

3. Improves Security

Documentation includes:

Firewall rules

Access control policies

Password policies

This helps maintain proper security and avoid unauthorized access.

4. Helps in Network Expansion

When adding new devices or upgrading the network, documentation shows:

Available IP addresses

Existing topology

Hardware details

This makes expansion easier and faster.

5. Useful for New Staff

If a new network administrator joins, they can understand the network structure quickly through proper documentation.

6. Supports Backup and Recovery

In case of system failure, documentation helps restore configurations correctly.

Network documentation is essential for efficient management, security, troubleshooting, and future growth of a network. It reduces errors, saves time, and ensures smooth operation.