Tasks

1.

Open Command Prompt or Terminal and use the ping command to test connectivity to www.spotify.com. Note the number of packets sent, received, and the average response time.

>Command Used: ping [www.spotify.com](https://www.spotify.com)

Packets Sent: 4

Packets Received: 4 (0% loss)

Average Response Time: 28 ms

2.

Use the traceroute (tracert on Windows, traceroute on Linux/Mac) command to trace the path from your computer to www.flipkart.com. List the number of hops and identify the IP address of the first and last hop.

> Command Used: tracert [www.flipkart.com](https://www.flipkart.com) (Windows)

Total Number of Hops: 11 hops

First Hop IP Address (Local Gateway): 192.168.1.1

Last Hop IP Address (Target Server): 163.53.78.114

3.

Simulate a connectivity issue by pinging a non-existent website (for example, ping www.fakezomato12345.com) and write down the error message you receive.<br><br><em><strong>Hint:</strong> Compare the output to a successful ping and describe the difference in your findings.</em>

>  Command Used: ping [www.fakezomato12345.com](https://www.fakezomato12345.com)

Error Message Received: Ping request could not find host [www.fakezomato12345.com](https://www.fakezomato12345.com). Please check the name and try again.

Comparison & Findings:

- Successful Ping: Correctly resolves the website name into an IP address, sends 32 bytes of data, and receives structured timing replies (e.g., Reply from... time=25ms).

- Failed Ping: Fails immediately at the DNS (Domain Name System) layer because the domain name does not exist. No IP address can be found, and no actual data packets are transmitted over the network.

4.

Imagine you are unable to access www.youtube.com from your system. Use both ping and traceroute commands to diagnose the issue and write a short report describing where the connection fails and what the possible reasons could be.

>1. Command Outputs (Simulated Findings)

- ping [www.youtube.com](https://www.youtube.com): Returns Request timed out with 100% packet loss.

- tracert [www.youtube.com](https://www.youtube.com):

  - Hops 1 through 3 (local router and internal ISP nodes) respond normally with low latency numbers.

  - Hop 4 and onwards return only asterisks (* * * Request timed out), completely halting before reaching YouTube's network.

2. Diagnosis & Possible Reasons

- Where the Connection Fails: The connection successfully leaves the local home network but breaks down right after entering the core infrastructure of the Internet Service Provider (ISP).

- Possible Reasons:

  - ISP Routing Issue: The ISP is experiencing a backbone configuration error or equipment failure that stops traffic from reaching external global networks.

  - Firewall / Content Filtering: A local firewall rule, corporate web filter, or parental control setting is intentionally intercepting and dropping packets destined for YouTube's IP block.

  - Regional Content Delivery Network (CDN) Outage: The local Google/YouTube edge cache server assigned to your region is down or temporarily unreachable.